Label 1 – Shallow Breathing

Clear reduction in amplitude of respiratory belt signals (lungs + diaphragm), without complete disappearance, with preserved or even increased respiratory rate.

Conditions:

- Amplitude reduction: 20–35% of reference amplitude (60-second analysis windows of stable recording).

- Duration: sustained ≥30 s.

- No desaturation (SpO₂ < 90% or SpO₂ drop ≥3–4% below baseline – baseline defined as mean from first 30–60 s of recording).

Label 2 – Irregular Breathing

Irregular breathing means that durations of consecutive breathing cycles and amplitudes vary unpredictably and without a clear pattern.

Conditions:

Inter-breath intervals (IBI) calculated as time between consecutive respiratory peaks (30–60 s segments).

- Coefficient of variation (CoV) of respiratory intervals.

- Root mean square of successive differences (RMSSD).

If CoV ≥ 0.3 or RMSSD ≥ 0.5 s → label: irregular breathing.

Note: Calculated separately for lungs and abdomen. No breathing pauses (≥10 s) allowed in the analyzed signal.

Label 3 – Slow Breathing (Bradypnea)

Reduced respiratory rate.

Mean respiratory rate (RR) ≤ 10 breaths/min sustained for ≥30 s.
60-second windows analyzed. If fewer than 10 breaths occur for at least 30 s → slow breathing.

Notes:

- Distinguish “slow + deep” vs “slow + shallow” using amplitude analysis (as in shallow breathing).

- If accompanied by SpO₂ drop ≥3% → label as slow breathing with desaturation (possible delay in SpO₂ signal).

Label 4 – Rapid Breathing (Tachypnea)

Increased respiratory rate.

Mean RR ≥20 breaths/min sustained for ≥30 s.
If more than 20 breaths for at least 30 s → rapid breathing.

Notes:

- Distinguish “fast + deep” vs “fast + shallow” via amplitude analysis.

- If SpO₂ drop ≥3% accompanies rapid breathing → label as rapid breathing with desaturation (possible SpO₂ delay).

Label 5 – Respiratory Asynchrony / Dyssynchrony

Lack of synchronization between lungs and diaphragm movements.

Possible cases:

- Mild asynchrony – phase shift

- Paradoxical breathing – movements in opposite directions (e.g., chest rises while abdomen falls)

Possible detection methods:

- Time–frequency methods (e.g., mathematical framework used by Tomislav)

- Phase coherence between diaphragm and chest signals in 60-second windows

Label 6 – Desaturation (Hypoxia)

Desaturation = episode where SpO₂ significantly decreases relative to baseline or crosses a low absolute threshold.

Criteria:

Episode = (ΔSpO₂ ≥3% or SpO₂ <90% relative to defined baseline) and duration ≥10 s.

Baseline: median SpO₂ from first 60s of stable recording.

Relation to other signals:

- ECG: HR often increases during desaturation

- BP: sudden increases/decreases may accompany acute SpO₂ drops

Label 7 – Apnea

Episode with near-complete absence of respiratory movements (no ventilation) for at least 10 s.

- Amplitude criterion: lungs ≤10% reference amplitude AND diaphragm ≤10% reference amplitude (≥90% reduction).

- Duration ≥10 s.

- Optional: accompanying SpO₂ drop (e.g., ≥3% within 30–60 s) – increases diagnostic certainty.

Note: With current measurements, it is not possible to distinguish central apnea (no respiratory effort) from obstructive apnea (respiratory effort present but no airflow – requires mask measurement).

Label 8 – Sigh

Single breath with clearly larger amplitude and/or prolonged inspiratory phase compared to normal cycles.

Conditions:

Cycle amplitude (max–min) ≥1.5 × median amplitude from previous 60 s of stable recording.

Additionally:

Frequency ≥7–10 sighs per 30 minutes.

Sighs may co-occur with:

- prolonged irregular breathing

- frequent SpO₂ drops

- significant chest–diaphragm asynchrony

- notable HR/BP changes